Contents

- ABP (Activation By Personalization)

- ABP (The Hard-coded Way)

Use Ctrl + Click to open this Notion docs page

# ABP (Activation By Personalization)

Here is a detailed explanation of ABP (Activation By Personalization), how it works under the hood, and why it is considered a "legacy" method that you should avoid now that you are upgrading your hardware.

1. The Core Concept: "The Pre-Made Key"

To understand ABP, imagine checking into a hotel.

- OTAA (The Standard Way): You walk up to the front desk, show your ID, and they give you a key card for a specific room. If you lose the key, you go back to the desk and get a new one.

- ABP (The Hard-coded Way): You skip the front desk entirely. You already have a metal key in your pocket that was cut specifically for Room 202. You walk straight to the room and open it.

In LoRaWAN terms, ABP means the device does not negotiate with the network server to join. It assumes it is already joined.

2. The Three Static Parameters

When you set up a device (like your Adeunis) using ABP, you must manually type three specific long strings of numbers into both the device and the Network Server (Vaculink dashboard). These must match perfectly:

- DevAddr (Device Address): A 32-bit ID (e.g., 26 01 1A F1). This acts like a short ID card. It is not unique worldwide, only unique within your specific network.

- NwkSKey (Network Session Key): This is the encryption key used to prove the data is authentic and ensures the network server can read the routing information.

- AppSKey (Application Session Key): This is the encryption key used to lock the actual payload (the sensor data). Only the final application (Vaculink) can decrypt this.

In ABP, these keys never change. They are burned into the device's memory.

3. The Workflow: "Wake Up and Shout"

Since the keys are already inside the device, the process is very simple:

- Power On: The Adeunis device wakes up.

- Encrypt: It takes its sensor data (e.g., "Vacuum Level: 50") and encrypts it using the hard-coded AppSKey.

- Sign: It signs the packet using the NwkSKey so the gateway knows it's valid.

- Transmit: It immediately broadcasts the message on a specific frequency (or hops frequencies).

- Sleep: It goes back to sleep.

- The Missing Step: Notice there was no "Hello, can I join?" message. The device just started talking. This is why the weak LG01 gateway liked ABP—it didn't have to do any complex processing to accept the device.

4. The Major Technical Risks (Why you should stop using it)

Now that you are buying a Dragino LPS8/DLOS8 (8-Channel Gateway), you should stop using ABP. Here is why:

- The "Frame Counter" Trap (Most Common Failure)

To prevent hackers from recording a message and playing it back later (Replay Attack), LoRaWAN devices count their messages (1, 2, 3... 100). The server tracks this count. If the server expects message #101 but receives message #50, it rejects it.

The Problem: In ABP, if your Adeunis device loses power or reboots, its internal counter often resets to 0.

The Result: The device sends message #1. The Server (Vaculink) expects message #101. The Server thinks this is a hack attempt and blocks the data.

The Fix: You have to manually log into the server and "reset frame counters," which is a maintenance nightmare.

- The "Truck Roll" Problem

If you ever want to change security keys or move the device to a different network provider, you cannot do it remotely.

Since the keys are hard-coded, you must physically drive to the location ("roll the truck"), plug a laptop into the Adeunis device, and reprogram it.

- Static Security

If a hacker manages to steal the keys from one of your devices, they have those keys forever. In OTAA (the better method), the keys are temporary "session" keys that change every time the device rejoins the network.

Summary Comparison

| Feature | ABP (Activation By Personalization) | OTAA (Over-The-Air Activation) |
| --- | --- | --- |
| Setup | Hard-code keys into device & server. | Device has a generic ID; Network gives it keys automatically. |
| Connection | Instant. No handshake. | Requires a "Join Request" & "Join Accept" handshake. |
| Security | Low (Static keys). | High (Dynamic, rotating keys). |
| Resilience | Bad. Frame counter resets cause data loss. | Good. Rejoining resets counters automatically. |
| Best For | Simple prototypes, Single-Channel Gateways. | Real deployments (Vaculink), 8-Channel Gateways. |

Verdict: Use ABP only if you are forced to use the old LG01. Once you get your 8-Channel Gateway, switch your Adeunis devices to OTAA.

# ABP (The Hard-coded Way)

If ABP is "The Hard-coded Way," OTAA is "The Conversation." It is the gold standard for LoRaWAN and is exactly how your Adeunis device was designed to work.

1. The Core Concept: "The Hotel Check-In"

Let’s revisit the hotel analogy to compare it with ABP.

- ABP: You walk straight to Room 202 with a key you’ve had in your pocket for years.

- OTAA: You walk into the hotel lobby. You don't have a room key yet. You go to the front desk and show your Passport (DevEUI) and a Secret Password (AppKey). The receptionist verifies who you are and creates a new, temporary key card just for this stay.

In OTAA, the device and the network negotiate the security keys at the moment the device turns on.

2. The Three "Root" Credentials

Before an OTAA device can join, you must program it (or it comes pre-programmed) with three parameters. Unlike ABP, these are identifiers, not session keys.

- DevEUI (Device EUI): A globally unique 64-bit ID. Think of this as the device's "MAC Address" or Serial Number. No two devices in the world should have the same DevEUI.

- JoinEUI (formerly AppEUI): This tells the network which "Application" server the device wants to talk to.

- AppKey (Application Key): This is the most critical piece of data. It is a 128-bit AES master secret key. It is known only to the device and the Network Server. It is never sent over the air. It is used to calculate the session keys.

3. The Workflow: "The Handshake"

This is the complex process that the weak LG01 gateway struggled with, but your new 8-Channel Gateway will handle easily.

Step 1: The Join Request (Uplink)

When you turn on the Adeunis device, it doesn't send sensor data yet. It sends a specific message called a Join Request.

Content: "Hello, I am DevEUI, and I want to join JoinEUI."

The Nonce: It also includes a random number (DevNonce) to ensure this request is fresh and not a replay of an old request.

The Signature: It signs this message using the AppKey so the server knows it's not a fake device.

Step 2: The Server Verification

The Network Server receives the message. It looks up the DevEUI in its database.

It checks: "Do I have a device with this ID?"

It checks: "Is the signature correct using the matching AppKey?"

If yes, it decides to let the device in.

Step 3: The Join Accept (Downlink)

The Server sends a message back to the device called Join Accept.

Timing: This happens exactly 5 or 6 seconds after the request. (This precise timing is why single-channel gateways fail—they miss the timing window).

Content: This message contains network settings (like which channels to use) and a random number from the server.

Encryption: This entire message is encrypted using the AppKey.

Step 4: Key Derivation (The Magic Trick)

This is the genius of OTAA.

The Device has the AppKey and the random numbers exchanged during the handshake.

The Server has the AppKey and the same random numbers.

Both sides run a mathematical calculation independently to generate the Session Keys (NwkSKey and AppSKey).

Result: Both the device and the server now have the keys to encrypt data, but the keys were never actually sent through the air. A hacker listening with an antenna sees nothing but random noise.

4. Why OTAA Solves the ABP Problems

- Solves the "Frame Counter" Issue

Remember how ABP devices get blocked if they lose power and their counter resets to 0?

OTAA Fix: Every time an OTAA device reboots, it performs a new "Join Request."

When the server accepts the new join, it automatically resets the frame counter to 0 on the server side.

Benefit: You can pull the battery out of your Adeunis device, put it back in, and it will just work. No manual server reset required.

- Solves the Security Issue

OTAA Fix: If you suspect a security breach, or if you just want to be safe, you can force the device to "Re-Join." It will generate brand new Session Keys.

Benefit: The keys are dynamic. They change every session.

- Roaming and Scalability

If you move your Vaculink setup to a different location or a different network provider, you don't need to plug a cable into the device. You just update the keys on the server, and the device will join the new network automatically over the air.

Summary for Vaculink

By using OTAA with your Adeunis device and an 8-Channel Gateway:

| Step | Action |
| --- | --- |
| Device Powers On | Sends "Join Request" (hopping frequencies) |
| 8-Channel Gateway | Hears the request (because it listens to all frequencies) |
| Server | Validates and sends "Join Accept" |
| Device | Generates keys and starts sending Vacuum data |

This is the professional standard. It is secure, robust against power failures, and requires much less maintenance than ABP.