Contents

- Cellular IoT Field Testing

- Cellular IoT Field Test Device (FTD) Specs & Operations

Use Ctrl + Click to open this Notion docs page

# Cellular IoT Field Testing

1. The "Coverage Myth": Smartphones vs. IoT

A common engineering mistake is assuming that if a smartphone has 4G signal, an IoT sensor will work. This is incorrect due to technical differences:

Smartphones use LTE Cat-4 (High power, high bandwidth).

IoT Sensors use NB-IoT or LTE-M (Low power, low bandwidth).

Frequency Bands: Operators often deploy IoT services on specific bands (e.g., Band 20 or Band 8) that offer better penetration but might not be the primary band used by phones.

Roaming: An IoT SIM might be restricted to specific towers that a consumer SIM is not.

2. The Testing Hardware: Adeunis FTD (Cellular)

Device Type: A dedicated network tester for NB-IoT and LTE-M.

Interface: Uses Bluetooth to display data on a Smartphone App (iOS/Android), allowing the hardware to remain compact and battery-efficient.

Power: Rechargeable via USB-C.

SIM Compatibility: Accepts standard Nano-SIM cards (Mono-operator or Multi-operator/Roaming).

3. Key Operation Modes

A. Network Scan (Blind Scan)

Function: The device scans the entire frequency spectrum to identify all available towers and technologies in the area, regardless of the SIM card inserted.

Output: A list of Operator IDs (PLMN), Technologies (NB-IoT/LTE-M), and Signal Strength bars.

Engineering Value: Helps select the best network operator for a specific geographic location before purchasing SIM contracts.

B. Network Test (Active Connection)

Function: Forces a connection using the inserted SIM credentials to send a data packet (UDP/TCP/Ping) to a server.

Engineering Value: Validates that the device can actually authenticate and transmit data, not just see the tower.

C. Radio Mapping (Site Survey)

Function: The user walks through a facility or city. The App correlates the GPS Location of the phone with the Signal Quality measured by the device.

Output: A heat map or report showing coverage quality at every specific installation point.

4. Critical Signal Metrics (The "Health Check")

When analyzing test results, engineers look at these specific values:

| Metric | Definition | Good Range | Critical Note |
| --- | --- | --- | --- |
| RSRP | Reference Signal Received Power. The raw signal strength. | > -100 dBm | Below -115 dBm is the "Danger Zone" (likely packet loss). |
| RSRQ | Reference Signal Received Quality. The quality of the signal relative to noise/interference. | > -10 dB | If RSRP is good but RSRQ is bad, there is high interference. |
| SNR | Signal-to-Noise Ratio. Clarity of the signal. | > 0 dB | Negative values mean the noise is louder than the signal. |
| ECL | Coverage Enhancement Level. (NB-IoT Only). | 0, 1, or 2 | Critical for Battery Life. (See below). |

5. The "ECL" Trap (NB-IoT Specific)

ECL (Coverage Enhancement Level) is a unique feature of NB-IoT that allows it to work deep underground.

ECL 0: Normal connection. Fast transmission. Low battery usage.

ECL 1: Weak signal. The device repeats the message multiple times to be heard. Medium battery usage.

ECL 2: Deep coverage (e.g., basements). The device repeats the message up to 128 times.

Engineering Consequence: While the device works in ECL 2, it consumes significantly more energy. A 10-year battery life calculation might drop to 1 year if the device is permanently in ECL 2.

Summary

Field Testing is mandatory. Never deploy cellular IoT sensors based on guesswork or smartphone coverage.

Check the ECL. For battery-powered applications, ensure devices are placed in locations with ECL 0 or 1. Avoid ECL 2 unless using external power.

Mapping Reports: Use the "Radio Mapping" mode to generate proof of coverage for client sign-off before installation.

# Cellular IoT Field Test Device (FTD) Specs & Operations

1. Hardware & Power Specifications

Battery Autonomy: Approximately 15 Days based on a standard workday usage (8-10 hours/day). This allows for extended field campaigns without daily charging logistics.

Charging Interface: USB-C Fast Charging. Note that full charge cycles can take up to 6 hours; overnight charging is recommended over "quick top-ups" in the field.

Global Compatibility: The device supports all major global frequency bands for NB-IoT and LTE-M. A single hardware unit can be deployed internationally by simply swapping the local Nano-SIM card.

2. Cost & Procurement Model

CAPEX vs. OPEX: The device follows a Capital Expenditure model (approx. €450 one-time purchase).

Software Costs: The companion Smartphone App (iOS/Android) is typically free and does not require a recurring subscription.

ROI Calculation: Compared to hiring external RF consultants (often >€1000/day), owning the tool allows internal teams to perform unlimited surveys, amortizing the cost quickly.

3. Operational Modes & Limitations

A. Supported Modes

Network Scan: Passive scanning of the environment to identify available operators and technologies (NB-IoT vs LTE-M) without needing to authenticate.

Network Test: Active connection using the inserted SIM to measure specific link quality metrics (RSRP, RSRQ, SNR, ECL).

Radio Mapping: Continuous logging of Signal Quality + GPS Location to create coverage heatmaps.

B. Technical Limitation (Traffic Testing)

Current Constraint: The device focuses on Radio Link Validation (Layer 1/2). It does not currently support higher-level traffic tests like Ping (Latency) or Throughput (Speed) testing.

Implication: It validates connectivity and signal strength, but not server response time. For low-bandwidth IoT applications (metering, status updates), this is usually sufficient.

4. Documentation & Reporting Workflow

Field testing is useless without data retention. The device supports a structured reporting workflow:

Data Export: Measurements can be exported in CSV (raw data) or Word (formatted reports).

Integration: Data can be pushed to an HTTP server in JSON format for integration with centralized planning tools.

Liability Protection: Generating a "Radio Mapping Report" before installation provides proof of coverage conditions at the time of deployment, serving as a baseline for future troubleshooting.

5. Specific Use Cases

Deep Indoor/Underground: Explicitly designed to validate coverage in basements, sewer pits, and utility cabinets where standard smartphones fail to connect.

Antenna Optimization: The device provides real-time feedback, allowing installers to micro-adjust antenna positions (e.g., moving an antenna 10cm or changing its orientation) to maximize Signal-to-Noise Ratio (SNR) before permanently fixing the equipment.

Summary

Tooling Strategy: For professional IoT deployment, a dedicated FTD is standard equipment. It bridges the gap between "blind" installation and fully certified RF engineering.

Maintenance: While battery life is excellent, the long charging time requires disciplined equipment management.

Data Value: The ability to map and export coverage data transforms a qualitative "it works" into a quantitative engineering record.