Contents

- LoRaWAN Technology & Strategy

- Why ChirpStack Is Mandatory in LoRaWAN Architecture

Use Ctrl + Click to open this Notion docs page

# LoRaWAN Technology & Strategy

1. Technology Overview: What is LoRaWAN?

Definition: A Low Power Wide Area Network (LPWAN) protocol designed for massive IoT connectivity.

Key Characteristics:

Long Range: Kilometers of coverage (10km+ rural, 1-3km urban).

Deep Penetration: Signals pass effectively through concrete, metal, and underground structures (better than cellular).

Low Power: Devices sleep 99% of the time, enabling battery life of 10-20 years.

Low Data Rate: Designed for small packets (sensor readings), not video or voice.

2. Network Models

Unlike cellular (which is always public), LoRaWAN offers flexible deployment models:

Public Network: Operated by a telecom company (like a mobile network). You pay a subscription per device.

Private Network: You buy and install your own Gateways. You own the network. No monthly fees. Ideal for factories, mines, or campuses.

Hybrid Network: A combination of both, allowing roaming between private sites and public coverage.

3. LoRaWAN vs. NB-IoT (The Comparison)

This is the most common strategic decision for engineers.

| Feature | LoRaWAN | NB-IoT (Cellular) |
| --- | --- | --- |
| Spectrum | Unlicensed (Free - ISM Band). | Licensed (Paid - Cellular Band). |
| Deployment | Anyone can build a network (Private). | Only Telecoms can build networks. |
| Battery Life | Superior (10+ years). | Good, but lower due to handshake overhead. |
| Cost | Low (Device + Gateway). | Higher (SIM cards + Monthly Fees). |
| Latency | High (Not real-time). | Low (Better for critical alerts). |
| Best For | Massive sensor fleets, rural/remote areas, private campuses. | Critical infrastructure, urban areas with existing cell towers. |

4. Advanced Capabilities

Satellite IoT: LoRaWAN signals can be received by Low Earth Orbit (LEO) satellites. This fills coverage gaps in deserts or oceans where no gateways exist.

Geolocation: Can approximate device location without GPS by using "Time Difference of Arrival" (TDOA) from multiple gateways (less accurate than GPS but uses far less battery).

Firmware Over-The-Air (FOTA): Supports Multicast, allowing you to send a firmware update to 1,000 devices simultaneously, rather than updating them one by one.

5. Key Use Cases

Asset Tracking: Monitoring non-powered assets (bins, pallets, livestock) where changing batteries is impossible.

Smart Cities: Street lighting, waste management, parking sensors, environmental monitoring.

Connected Worker: Safety wearables in mines/construction (SOS buttons, geofencing, fall detection) using private networks for reliability.

Utilities: Water and gas metering (deep indoor/underground penetration is crucial here).

Summary

LoRaWAN is the de facto standard for "Massive IoT" where battery life and cost are more important than speed.

Private Networks give enterprises total control over their data and coverage, independent of telecom operators.

Hybrid Connectivity (Terrestrial + Satellite) is solving the "Global Coverage" challenge for logistics and remote monitoring.

# Why ChirpStack Is Mandatory in LoRaWAN Architecture

The Reality Check: Can you really bypass ChirpStack?

The short answer is: Technically, No.

You cannot go directly from a LoRaWAN sensor to a custom MQTT backend without a "Network Server" in the middle.

The Analogy:

Imagine the sensors are people speaking a secret code (LoRaWAN encryption).

The Gateway is just a microphone. It hears the noise, but it doesn't know the code.

ChirpStack is the Translator. It has the "Code Book" (the secret keys) to turn the noise into English (JSON).

MQTT is the Messenger who carries that English translation to your Vaculink 3.0 portal.

If you "bypass" ChirpStack, you have a microphone (Gateway) and a Messenger (MQTT), but no one to translate the secret code. The messenger will just be carrying gibberish.