Contents

- NO and NC Contacts

- NO (Normally Open) and NC (Normally Closed) contacts.

Use Ctrl + Click to open this Notion docs page

# NO and NC Contacts

Working Principle, Logic Flow, and Fail-Safe Behavior

NO (Normally Open) and NC (Normally Closed) contacts.

These terms describe the default state of a switch, relay, or sensor when it is at rest—meaning no power is applied, no button is pressed, and no physical force is acting on it.

1. NO (Normally Open)

The Concept:

Imagine a drawbridge that is raised up. Cars (electricity) cannot cross the river. The path is broken. This is the "Normal" state.

At Rest (Default): The circuit is incomplete (Open). Current cannot flow.

Activated (Action): When you press the button or trigger the sensor, the "bridge" lowers and connects the two sides. The circuit closes, and current flows.

Release: When you let go, a spring pushes the contacts apart, breaking the connection again.

Logic Flow:

0 (Rest)

→

→

1 (Active)

Common Examples:

A Doorbell: It sits silent (disconnected) until you push the button to complete the circuit and ring the bell.

Computer Keyboard Keys: The circuit is open until you press a key to send a signal.

Light Switch: In the "Off" position, the circuit is physically broken (Open).

2. NC (Normally Closed)

The Concept:

Imagine a solid bridge across a river. Cars (electricity) are constantly crossing it. The path is complete. This is the "Normal" state.

At Rest (Default): The circuit is connected (Closed). Current flows freely through the switch.

Activated (Action): When you press the button or trigger the sensor, you physically break the connection (lift the bridge). The circuit opens, and the current stops flowing.

Release: When you let go, a spring pushes the contacts back together, restoring the flow of electricity.

Logic Flow:

1 (Rest)

→

→

0 (Active)

Common Examples:

Emergency Stop Button (E-Stop): The machine runs only while the current flows through the closed switch. If you hit the button, it breaks the circuit, cutting power immediately.

Refrigerator Light: The button is held "pressed" (Open circuit/Light Off) by the closed door. When you open the door, the button is released to its "Normal" state (Closed circuit), and the light turns on.

3. The Critical Difference: Wire Breaks (Fail-Safe)

The most important engineering distinction between NO and NC is how the system behaves if a wire is accidentally cut.

Scenario: A wire is cut by accident.

If using NO (Normally Open):

The system sees an "Open" circuit.

This looks exactly the same as the "At Rest" state.

Result: The system thinks everything is fine. It will never know the sensor is broken. If an emergency happens, the signal cannot get through.

If using NC (Normally Closed):

The system expects to see a continuous flow of current (Closed).

When the wire is cut, the flow stops (Open).

This looks exactly the same as the "Activated" state.

Result: The system reacts immediately. It thinks the button was pressed or the alarm triggered. This is called Fail-Safe because if it fails (wire cut), it fails into a "safe" or "alerted" mode.

Summary Table

| Feature | NO (Normally Open) | NC (Normally Closed) |
| --- | --- | --- |
| Symbol | _/ _ (Gap in line) | / (Line connected) |
| Default State | Disconnected (OFF) | Connected (ON) |
| When Activated | Connects (turns ON) | Disconnects (turns OFF) |
| Electrical Flow | No flow until triggered. | Constant flow until triggered. |

# NO (Normally Open) and NC (Normally Closed) contacts

These terms describe the default state of a switch, relay, or sensor when it is at rest—meaning no power is applied, no button is pressed, and no physical force is acting on it.

1. NO (Normally Open)

The Concept:

Imagine a drawbridge that is raised up. Cars (electricity) cannot cross the river. The path is broken. This is the "Normal" state.

At Rest (Default): The circuit is incomplete (Open). Current cannot flow.

Activated (Action): When you press the button or trigger the sensor, the "bridge" lowers and connects the two sides. The circuit closes, and current flows.

Release: When you let go, a spring pushes the contacts apart, breaking the connection again.

Logic Flow:

0 (Rest)

→

→

1 (Active)

Common Examples:

A Doorbell: It sits silent (disconnected) until you push the button to complete the circuit and ring the bell.

Computer Keyboard Keys: The circuit is open until you press a key to send a signal.

Light Switch: In the "Off" position, the circuit is physically broken (Open).

2. NC (Normally Closed)

The Concept:

Imagine a solid bridge across a river. Cars (electricity) are constantly crossing it. The path is complete. This is the "Normal" state.

At Rest (Default): The circuit is connected (Closed). Current flows freely through the switch.

Activated (Action): When you press the button or trigger the sensor, you physically break the connection (lift the bridge). The circuit opens, and the current stops flowing.

Release: When you let go, a spring pushes the contacts back together, restoring the flow of electricity.

Logic Flow:

1 (Rest)

→

→

0 (Active)

Common Examples:

Emergency Stop Button (E-Stop): The machine runs only while the current flows through the closed switch. If you hit the button, it breaks the circuit, cutting power immediately.

Refrigerator Light: The button is held "pressed" (Open circuit/Light Off) by the closed door. When you open the door, the button is released to its "Normal" state (Closed circuit), and the light turns on.

3. The Critical Difference: Wire Breaks (Fail-Safe)

The most important engineering distinction between NO and NC is how the system behaves if a wire is accidentally cut.

Scenario: A wire is cut by accident.

If using NO (Normally Open):

The system sees an "Open" circuit.

This looks exactly the same as the "At Rest" state.

Result: The system thinks everything is fine. It will never know the sensor is broken. If an emergency happens, the signal cannot get through.

If using NC (Normally Closed):

The system expects to see a continuous flow of current (Closed).

When the wire is cut, the flow stops (Open).

This looks exactly the same as the "Activated" state.

Result: The system reacts immediately. It thinks the button was pressed or the alarm triggered. This is called Fail-Safe because if it fails (wire cut), it fails into a "safe" or "alerted" mode.

Summary Table

| Feature | NO (Normally Open) | NC (Normally Closed) |
| --- | --- | --- |
| Symbol | _/ _ (Gap in line) | / (Line connected) |
| Default State | Disconnected (OFF) | Connected (ON) |
| When Activated | Connects (turns ON) | Disconnects (turns OFF) |
| Electrical Flow | No flow until triggered. | Constant flow until triggered. |
| Wire Break | System sees "OFF" (No Alert). | System sees "ON" (False Alert). |
| Best Use | Start buttons, Doorbells. | Stop buttons, Safety alarms. |