Contents

- PLC Fundamentals: Architecture & Operation

- Why PLCs Exist: Reliability, Determinism, and Control

Use Ctrl + Click to open this Notion docs page

# PLC Fundamentals: Architecture & Operation

1. What is a PLC?

Definition: An industrial-grade digital computer designed for manufacturing and automation.

The Purpose: To execute control logic (If X happens, do Y) with extreme reliability and speed.

Evolution: They replaced massive walls of electromechanical relays.

Old Way (Relays): To change logic, you had to physically rewire the panel.

New Way (PLC): To change logic, you just update the software code.

2. System Architecture (The Anatomy)

A PLC has three distinct sections, just like a human body.

A. Input Modules (The Senses)

Function: Detects the real world.

Digital Inputs: On/Off signals (e.g., Float Switches, Pushbuttons).

Analog Inputs: Variable signals (e.g., Vacuum Pressure Sensor 4-20mA, Thermistors).

Key Feature: Optical Isolation. The inputs protect the sensitive CPU from high voltage spikes in the field.

B. The CPU (The Brain)

Function: Stores the program and makes decisions.

Components: Microprocessor + Memory.

Communication: Ports for Modbus, Ethernet/IP, or Profinet to talk to Gateways (like your Ewon).

C. Output Modules (The Muscles)

Function: Executes the command.

Digital Outputs: Turn things On/Off (e.g., Solenoid Valves, Pilot Lights, Motor Starters).

Analog Outputs: Control speed or position (e.g., Variable Frequency Drive for a pump).

3. The PLC Operation Cycle (The "Scan")

Unlike a normal computer that waits for a mouse click, a PLC runs in a continuous, lightning-fast loop called a Scan Cycle.

Input Scan: Take a snapshot of all sensors (Is the button pressed? Is pressure low?).

Program Scan: Run the logic code (Ladder Logic) against that snapshot.

Program Execution: Calculate what should happen.

Output Update: Send the electrical signals to the pumps/valves.

Housekeeping: Check for internal faults and communicate with the SCADA/Vaculink.

Speed: This whole cycle happens in milliseconds.

4. Control Strategies inside a PLC

A. Discrete Control (On/Off)

Logic: Simple thresholds.

Example: "If Vacuum Pressure < -0.50 bar, Turn Pump ON."

Scheduling: "Only run the discharge pump between 6:00 AM and 10:00 PM."

B. PID Control (Analog Loop)

Logic: Smooth adjustment (Proportional-Integral-Derivative).

Example: Instead of turning a pump 100% ON or OFF, the PLC tells the VFD (Variable Frequency Drive) to run at 43% speed to maintain exactly -0.60 bar pressure.

C. Optimization (Duty/Standby)

Logic: Wear leveling.

Example: "Pump A ran for 10 hours yesterday. Today, use Pump B as the primary pump to keep wear even."

🏗️ Analysis for Vaculink 3.0 System

How does the PLC fit into your architecture?

- The "Local Brain" vs. "Cloud Brain"

The PLC (Local): It lives at the Vacuum Station. It controls the pumps Directly.

Why? If the Internet cuts out, the PLC keeps running. The toilets still flush. This is Critical Infrastructure.

Vaculink 3.0 (Cloud): It lives on the Internet. It watches the PLC.

Role: Supervisory. You use Vaculink to change the settings of the PLC (e.g., "Change target pressure to -0.7 bar"), but you never let the Cloud directly switch the pump on/off in real-time (too much latency).

- The Ewon Connection

In your dashboard screenshots, we saw "Ewon" devices.

The Ewon Gateway is the bridge. It speaks Modbus/TCP to the PLC and MQTT/HTTPS to Vaculink 3.0.

Data Flow: Sensor

→

→

PLC Input

→

→

PLC Memory

→

→

Ewon reads Memory

→

→

Vaculink Dashboard.

# Why PLCs Exist: Reliability, Determinism, and Control

The Birth of the PLC

1. The Problem: "Click Clack Logic"

Before 1968, automation was done using thousands of physical Relays wired together.

The Sound: Factories were noisy because of the constant clicking of magnets closing contacts.

The Nightmare: If you wanted to change the logic (e.g., "Wait 5 seconds instead of 2"), you had to shut down the factory and physically rewire the panel.

Reliability: Relays have moving parts. They wear out. Finding a broken relay in a wall of 5,000 relays was impossible.

2. The Father: Dick Morley

The Man: An engineer who hated the boredom of repetitive consulting work.

The Idea: On New Year's Day 1968, he drafted a design for a "Black Box" that could replace relays.

The Philosophy: It had to be Rugged.

No fans (they break).

Sealed metal case (to stop dust/oil).

Heat fins (passive cooling).

Note: This is exactly the same philosophy as your Adeunis IP68 sensors. Industrial gear must survive where humans cannot.

3. The Invention: The Modicon 084

Name: MOdular DIgital CONtroller.

The Trick: Morley knew factory electricians were scared of "Computers." So, he didn't call it a computer. He called it a Programmable Controller.

The Language: He designed Ladder Logic to look exactly like the old relay wiring diagrams.

Result: Electricians could program it without learning "Code." They just drew the circuits on a screen instead of with copper wire.

🏗️ Analysis for Vaculink 3.0 System

Why does a history lesson from 1968 matter for your 2026 Cloud Dashboard?

1. The "Modbus" Connection

Modicon (Morley's company) invented a language for their PLCs to talk to each other.

They called it Modbus.

Your System: Your Ewon Gateway speaks Modbus TCP/IP to read data from the Vacuum Station PLC.

The Legacy: Every time your dashboard updates a vacuum pressure value, you are using the protocol invented by the company Dick Morley founded.

2. "Deterministic" vs. "Probabilistic"

Morley designed the PLC to have No Interrupts.

PLC (Deterministic): It scans inputs, does logic, and sets outputs in a perfect loop. It never pauses to "think" or "load." It is 100% predictable.

IoT/Cloud (Probabilistic): Your LoRaWAN sensor might miss a packet. The Cloud might have 500ms latency.

The Lesson: This is why Vaculink 3.0 (Cloud) must never try to replace the PLC (Local). The PLC guarantees safety (Deterministic). The Cloud provides insight (Data).

3. Reliability Culture

The video mentions General Motors (GM) buying the first PLCs.

Context: GM needed machines that could run 24/7 for years without rebooting.

Your Context: A Vacuum Sewer System is the same. It cannot stop.

Architecture: By keeping the PLC at the station and using Adeunis for the remote pits, you are respecting this culture of reliability.

Teacher's Summary

Dick Morley saved the world from "Click Clack" relay hell.

Modicon gave us the Modbus protocol, which is the universal language of industrial automation today.

Ladder Logic exists because it mimics the physical relays of the past.